Alesis D4 Midi Implementation

April 3, 1992

D4XE "D4"§ Software Version 1.04

Including

September 22, 1995

D5XE "D5"§ V1.01

D5XE "D5"§ differences are noted in italics.

TRANSMITTED DATA

Channel Messages

| Status | Second | Third | Description |
| --- | --- | --- | --- |
| 1001 nnnn | 0kkk kkkk | 0vvv vvvv | Note On |
|  |  |  | kkk kkkk = 0-127 |
|  |  |  | (61 Key Window + Root Note) |
|  |  |  | vvv vvvv = 1-127 |
|  |  |  | vvv vvvv = 0 = Note Off |

Universal System Exclusive Messages

(Transmitted on power upXE "power up"§ and when Inquiry Request is received)

| Byte | Description |
| --- | --- |
| 1111 0000 | Exclusive Status |
| 0111 1110 | Non-Real Time Message ID |
| 0uuu uuuu | Universal Sysex Channel (Same as D4XE "D4"§ MidiXE "MIDI"§ Channel) |
| 0000 0110 | General Information Sub-ID |
| 0000 0010 | Inquiry Message Identity Reply |
| 0000 0000 | Manufacturers System Exclusive ID Code |
| 0000 0000 | Alesis = 00H,00H,0EH |
| 0000 1110 |  |
| 0000 0110 | Device Family Code LSB (D4XE "D4"§ = 6 D5XE "D5"§ = 13H) |
| 0000 0000 | Device Family Code MSB |
| 0000 0000 | Device Family Member LSB (D4XE "D4"§ = 0) |
| 0000 0000 | Device Family Member MSB |
| 0qqq qqqq | Software Version LSB |
| 0qqq qqqq | Software Version MSB |
| 0rrr rrrr | Software Revision LSB |
| 0rrr rrrr | Software Revision MSB |
| 1111 0111 | EOX |

D4XE "D4"§/D5XE "D5"§ System Exclusive Messages

| Byte | Description |
| --- | --- |
| 1111 0000 | Exclusive Status |
| 0000 0000 | Manufacturers System Exclusive ID Code |
| 0000 0000 | Alesis = 00H,00H,0EH |
| 0000 1110 |  |
| 0000 0110 | Device ID (D4XE "D4"§ = 6) |
| 0000 nnnn | MidiXE "MIDI"§ Channel |
| 00oo oooo | Opcode (see Sysex section for opcode definitions and data structures) |
| 0ddd dddd | Data |
| ---- ---- | --- |
| 1111 0111 | EOX |

RECOGNIZED RECEIVE DATA

Channel Messages

| Status | Second | Third | Description |
| --- | --- | --- | --- |
| 1001 nnnn | 0kkk kkkk | 0vvv vvvv | Note On |
|  |  |  | kkk kkkk = 0-127 |
|  |  |  | (61 Key Window + Root Note) |
|  |  |  | vvv vvvv = 1-127 |
| 1011 nnnn | 0000 0110 | 0vvv vvvv | Data Entry MSB *1 |
| 1011 nnnn | 0000 0111 | 0vvv vvvv | Volume *1 |
| 1011 nnnn | 0110 0000 | 0xxx xxxx | Data Increment *1 |
|  |  |  | xxx xxxx = Ignored |
| 1011 nnnn | 0110 0001 | 0xxx xxxx | Data Decrement *1 |
|  |  |  | xxx xxxx = Ignored |
| 1011 nnnn | 0110 0010 | 0vvv vvvv | Non-Registered Parameter Number LSB *1 |
| 1011 nnnn | 0110 0011 | 0vvv vvvv | Non-Registered Parameter Number MSB *1 |
|  |  |  | (see Parameter section for specific values) |
| 1011 nnnn | 0110 0100 | 0vvv vvvv | Registered Parameter Number LSB *1 |
| 1011 nnnn | 0110 0101 | 0vvv vvvv | Registered Parameter Number MSB *1 |
|  |  |  | (see Parameter section for specific values) |
| 1011 nnnn | 0111 1001 | 0xxx xxxx | ResetXE "reset"§ All Controllers *1 |
| 1011 nnnn | 0111 1010 | 0000 0000 | Local Control Off |
| 1011 nnnn | 0111 1010 | 0111 1111 | Local Control On |
| 1011 nnnn | 0111 1100 | 0xxx xxxx | Omni Mode Off |
| 1011 nnnn | 0111 1101 | 0xxx xxxx | Omni Mode On |
| 1100 nnnn | 0ppp pppp | ---- ---- | Program change |
| 1110 nnnn | 0mmm mmmm | 0nnn nnnn | Pitch Bend Change *1 |

*1  Recognized only if CONTROLLERS is set to ON in MIDIXE "MIDI"§ page

Universal System Exclusive Messages

| Byte | Description |
| --- | --- |
| 1111 0000 | Exclusive Status |
| 0111 1110 | Non-Real Time Message ID |
| 0uuu uuuu | Universal Sysex Channel (ignored if OMNI = ON, 7FH = any channel) |
| 0000 0110 | General Information Sub-ID |
| 0000 0001 | Identity Request |
| 1111 0111 | EOX |

D4XE "D4"§ System Exclusive Messages

| Byte | Description |
| --- | --- |
| 1111 0000 | Exclusive Status |
| 0000 0000 | Manufacturers System Exclusive ID Code |
| 0000 0000 | Alesis = 00H,00H,0EH |
| 0000 1110 |  |
| 0000 0110 | Device I.D. (D4XE "D4"§ = 6 D5XE "D5"§ = 13H) |
| 0uuu uuuu | MidiXE "MIDI"§ Channel 7FH = Any channel |
| 00oo oooo | Opcode (see Sysex section for opcode definitions and data structures) |
| 0ddd dddd | Data |
| ---- ---- | --- |
| 1111 0111 | EOX |

| Byte | Description |
| --- | --- |
| 1111 0000 | Exclusive Status |
| 0000 0000 | Manufacturers System Exclusive ID Code |
| 0000 0000 | Alesis = 00H,00H,0EH |
| 0000 1110 |  |
| 0000 0110 | Device I.D. (D4XE "D4"§ = 6 D5XE "D5"§ = 13H) |
| 0uuu uuuu | MidiXE "MIDI"§ Channel 7FH = Any channel |
| 01oo oooo | Data Request (see Sysex section for opcode definitions) |
| 1111 0111 | EOX |

RECOGNIZED CONTINUOUS CONTROLLER PARAMETERS

The most reliable way to edit a parameter using a registered or non-registered parameter number is to first transmit the parameter number MSB. In the D4XE "D4"§ this is always zero, and it only needs to be sent once to initialize registered or non-registered parameter number reception. If both MSB and LSB messages have not been sent, any data entry, data increment, or data decrement messages will alter the parameter selected by the user as indicated by the cursor position in the display.

When a valid parameter number has been set up for editing, the display and cursor will be updated just as though the parameter had been selected from the front panel. (If an associated display exists!) Since it is possible for the user to select a new parameter for editing from the front panel between a series of data entry, data increment, or data decrement messages, it is strongly recommended that the parameter number LSB always be re-sent immediately before transmitting any of these controllers. This will insure that they are always acting on the intended parameter.

The actual 7 bit values for continuous controller messages 96 (data increment) and 97 (data decrement) are ignored. Likewise, continuous controller message 38 (data entry LSB) is ignored. To find the value of the 7 bit data entry MSB message (controller 6) to transmit in order to select a real-life setting, multiply the desired value by 127, and divide the result by the maximum allowable value for that parameter.

For example, the data entry MSB value used to set the Preview note's volume to 50 is 64 because:

50 x 127 / 99 = 64

The data entry MSB value used to set the Drum set's Root Note's to 44 is 83 because:

44 x 127 / 67 = 83

Registered Parameter Numbers for Controllers 100 (LSB) and 101 (MSB)

| MSB | LSB | Description |
| --- | --- | --- |
| 00H | 00H | Pitch bend sensitivity. (maximum range = +/- 12 semitones) |

Non-Registered Parameter Numbers for Controllers 98 (LSB) and 99 (MSB)

Trigger Parameters:

| MSB | LSB | Description | Range |
| --- | --- | --- | --- |
| 00H | 00H | Select active trigger. | 0-11 |
| 00H | 01H | Active trigger v-curve. | 0-7 |
| 00H | 02H | Active trigger note number. | 0 to 60 + Root Note |
| 00H | 03H | Active trigger gain. | 0-99 |
| 00H | 04H | Active trigger cross-talk. | 0-99 |
| 00H | 05H | Active trigger decay. | 0-99 |
| 00H | 06H | Active trigger noise floor. | 0-99 |
| 00H | 07H | Ignored |  |

Preview note specific:

| MSB | LSB | Description | Range |
| --- | --- | --- | --- |
| 00H | 08H | Preview note drum bank | Kik, Snr, Cym, Tom, Prc, Efx |
|  |  |  | Kik, Snr, Tom Cym, Prc, Efx, Rnd |
| 00H | 09H | Preview note drum sound. | Bank dependent |
| 00H | 0AH | Preview note coarse tune. | -4 to +3 semitones |
| 00H | 0BH | Preview note fine tune. | 0 to +99 cents |
| 00H | 0CH | Preview note volume. | 0 to 99 |
| 00H | 0DH | Preview note pan. | <3, <2, <1, <>, 1>, 2>, 3> |
| 00H | 0EH | Preview note output pair. | MAIN, AUX |
| 00H | 0FH | Preview note assign group. | MULTI, SINGLE, GROUP1, GROUP2 |

MidiXE "MIDI"§ parameters:

| MSB | LSB | Description | Range |
| --- | --- | --- | --- |
| 00H | 10H | Drumset root note | 0-67 |
| 00H | 11H | MidiXE "MIDI"§ channel. | 1-16 |
| 00H | 12H | MidiXE "MIDI"§ THRU. | OFF, ON, FLO |
| 00H | 13H | Program change enable. | OFF, ON |
| 00H | 14H | Controller enable. | OFF, ignored if already off! |
| 00H | 15H | Program table source. | 0 to 127 |
| 00H | 16H | Program table destination. | 0 to 20 |
| 00H | 17H | Ignored. MIDIXE "MIDI"§ LOCAL | On, Off |

Miscellaneous:

| MSB | LSB | Description | Range |
| --- | --- | --- | --- |
| 00H | 18H | Note Chase enable | Off, On |
| 00H | 19H | Quietly select preview note. | 0 to 60 + Root Note |
| 00H | 1AH | Flange Rate. | 0 = fast, 127 = slow |
| 00H | 1BH | Footswitch mode. | Hi Hat Pedal, Drumset Advance |
| 00H | 1CH | Footswitch close note. | 0 to 60 + Root Note |
| 00H | 1DH | Footswitch held note. | 0 to 60 + Root Note |
| 00H | 1EH | Ignored |  |
| 00H | 1FH | Ignored |  |

Drumset Management:

| MSB | LSB | Description | Range |
| --- | --- | --- | --- |
| 00H | 20H | Drumset name char 1 | 16-127 |
| 00H | 21H | Drumset name char 2 | 16-127 |
| 00H | 22H | Drumset name char 3 | 16-127 |
| 00H | 23H | Drumset name char 4 | 16-127 |
| 00H | 24H | Drumset name char 5 | 16-127 |
| 00H | 25H | Drumset name char 6 | 16-127 |
| 00H | 26H | Drumset name char 7 | 16-127 |
| 00H | 27H | Drumset name char 8 | 16-127 |
| 00H | 28H | Drumset name char 9 | 16-127 |
| 00H | 29H | Drumset name char 10 | 16-127 |
| 00H | 2AH | Drumset name char 11 | 16-127 |
| 00H | 2BH | Drumset name char 12 | 16-127 |
| 00H | 2CH | Drumset name char 13 | 16-127 |
| 00H | 2DH | Drumset name char 14 | 16-127 |
| 00H | 2EH | Store Drumset destination | 0-20 |
| 00H | 2FH | Store Drumset | False / True On the D5XE "D5"§ Asserting this acts as would the store button. |
| 00H | 30H | Recall Alesis Drumset srce. | 0 to 20 |
| 00H | 31H | Recall Alesis Drumset dest. | 0 to 20 |
| 00H | 32H | Recall all Alesis Drumsets. | False / True On the D5XE "D5"§ first assert this, then press the store button (or assert Store Drumset) |
| 00H | 33H thru 7FH | Ignored. |  |

SYSTEM EXCLUSIVE OPCODES AND DATA STRUCTURES

General Format:

All transmitted and received D4XE "D4"§ system exclusive messages follow this template:

On the D4XE "D4"§- F0H,00H,00H,0EH,06H,cc,qq,DATA	,F7H

On the D5XE "D5"§-F0H,00H,00H,0EH,13H,cc,qq,DATA	,F7H

If the message originated from the D4XE "D4"§/D5XE "D5"§ the sixth byte "cc" will always be the same as the midiXE "MIDI"§ channel. If an originator wishes to send a system exclusive message to any D4XE "D4"§ regardless of what midiXE "MIDI"§ channel is selected, "cc" should be set to 7FH. In addition, a D4XE "D4"§/D5XE "D5"§ set to OMNI will process all system exclusive messages it receives.

The seventh byte "qq" is the opcode. If bit 6 of this opcode is set the message is a data request and the opcode is always followed immediately by an end-of-exclusive status byte.  Opcodes with bit 6 cleared are data dump headers that identify the type of data that is to follow.  In either case, the lower six bits of the opcode represent the same data structure, whether the message is a data request or a data dump.

Most system exclusive messages on the D4XE "D4"§ include a checksum byte after the data immediately before the F7H end-of-exclusive status byte. This checksum is the modulo 128 addition of all the data bytes contained in the message starting with the byte immediately following the opcode.

| Opcode | Description | Length (Total bytes) | Checksum |
| --- | --- | --- | --- |
| 0000 0000 | System Info | 11 | No |
| 0000 0001 | Drumset Data for Edit Buffer | 343 | Yes |
| 0000 0010 | Old Trigger Setup (version 1.01 only) | 33 | Yes |
| 0000 0011 | Program Change Table | 137 | Yes |
| 0000 0100 | Display Message | 40 | No |
| 0000 0101 | New Trigger Setu | 69 | Yes |
| 0000 0110 | Reserved / Unused | x | x |
| ---- ---- | --- | x | x |
| ---- ---- | --- | x | x |
| 0001 1111 | Reserved / Unused | x | x |
| 001p pppp | Drumset Data for Drumset 0-20 | 343 | Yes |
| 0011 0101 | Reserved / Unused | x | x |
| ---- ---- | --- | x | x |
| ---- ---- | --- | x | x |
| 0011 1111 | Reserved / Unused | x | x |
| 01qq qqqq | Requests for opcodes 0-63 | 8 | No |

When a "SYSTEM (ALL)" bulk dump is initiated from the front panel the D4XE "D4"§'s entire memory contents get transmitted out MIDIXE "MIDI"§ in a series of 25 system exclusive messages. This series begins with the Program Change Table message, and is followed by the Trigger Setup message, the Edit Buffer message, 21 consecutive Single Set messages (beginning with Drumset 0), and finally the System Info message.

Note that it is possible in some systems to overflow the D4XE "D4"§'s midiXE "MIDI"§ receive buffer when sending data for more than one drumset. If this happens the transmitting device should pause for at least 250 milliseconds every 256 bytes to insure that the receiving D4XE "D4"§ has enough time to manage drumset memory.

Message 0 / System Info

Data Format (following header):

| Byte | Bit Field | Description |
| --- | --- | --- |
| 7 | 0000 0000 | Opcode = System Info |
| 8 | 0bbb bbbb | System Byte (see specific bit definitions following) |
| 9 | 0000 nnnn | MidiXE "MIDI"§ Channel (0-15) |
| 10 | 000s ssss | Currently selected drumset (0-20) |
| 11 | 1111 0111 | EOX |

System Byte Bit Definitions:

| Bit | Function | Polarity |
| --- | --- | --- |
| 0 | Omni Off | Set if omni mode is off. |
| 1 | MidiXE "MIDI"§ Thru Enable | Set if MIDIXE "MIDI"§ thru is enabled. |
| 2 | Program Change Enable | Set if program change receive is enabled. |
| 3 | Controllers Enable | Set if MIDIXE "MIDI"§ controllers are enabled. |
| 4 | Drumset Edited | Set if drumset in edit buffer has changed. |
| 5 | Footswitch Mode | 0 = Hi Hat, 1 = Drumset Advance |
| 6 | Note Chase Enable | Set if Note Chase is on. |

Message 1 / Edit Buffer

Data Format (following header):

| Byte | Bit Field | Description | Range |
| --- | --- | --- | --- |
| 7 | 0000 0001 | Opcode = Edit Buffer |  |
| 8 | 0aaa aaaa | Drumset name character 1 | (ascii) |
| 9 | 0aaa aaaa | Drumset name character 2 | (ascii) |
| --- | ---- ---- | --- | --- |
| --- | ---- ---- | --- | --- |
| 21 | 0aaa aaaa | Drumset name character 14 | (ascii) |
| 22 | 0mmm mmmm | Drumset Root Note | (0-67) |
| 23 | 00nn nnnn | Footswitch "closing" note | (0-60) |
| 24 | 00nn nnnn | Footswitch "held" note | (0-60) |
| 25 | 00nn nnnn | Trigger 1 note number | (0-60) |
| 26 | 00nn nnnn | Trigger 2 note number | (0-60) |
| --- | ---- ---- | --- | --- |
| --- | ---- ---- | --- | --- |
| 36 | 00nn nnnn | Trigger 12 note number | (0-60) |
| 37-41 | ---- ---- | Data packet for note 0 | (see below for packet definitions) |
| 42-46 | ---- ---- | Data packet for note 1 | --- |
| --- | ---- ---- | --- | --- |
| --- | ---- ---- | --- | --- |
| 337-341 | ---- ---- | Data packet for note 60 | --- |
| 342 | 0xxx xxxx | Checksum | (0-127) |
| 343 | 1111 0111 | EOX |  |

Note Data Packet Definitions (5 bytes per note):

| Byte | Bit Field | Description |
| --- | --- | --- |
| 0 | 0vvv vvvv | v = volume (0-99) |
| 1 | 0ppp obbb | p = panning: 0=left, 3=center , 6=right o = output: 0=main, 1 = aux b = drum bank: (0-6) Kik, Snr, Cym, Tom, Prc, Efx {0-8} Kik, Snr, Tom, Hat, Cym, Prc, Efx, Rnd |
| 2 | 0nnn nnnn | n = drum number (0-99, drum bank specific) |
| 3 | 0fff ffff | f = fine tuning (0-99 cents) |
| 4 | 0xxg gccc | x = reserved, always 0 g = assign group (0-3): 0=multi, 1=single, 2=group 1, 3=group 2 c = coarse tuning (-4,+3 semitones) |

Message 2 / Old Trigger Setup  (version 1.01 only)

Data Format (following header):

| Byte | Bit Field | Description | Range |
| --- | --- | --- | --- |
| 7 | 0000 0010 | Opcode = Trigger Setup |  |
| 8 | 000t tttt | Trigger 1 gain | (0-99) |
| 9 | 000t tttt | Trigger 2 gain | (0-99) |
| --- | ---- ---- | --- | --- |
| --- | ---- ---- | --- | --- |
| 19 | 000t tttt | Trigger 12 gain | (0-99) |
| 20 | 000t tttt | Trigger 1 type | (0-25) |
| 21 | 000t tttt | Trigger 2 type | (0-25) |
| --- | ---- ---- | --- | --- |
| --- | ---- ---- | --- | --- |
| 31 | 000t tttt | Trigger 12 type | (0-25) |
| 32 | 0xxx xxxx | Checksum | (0-127) |
| 33 | 1111 0111 | EOX |  |

Message 3 / Program Table

Data Format (following header):

| Byte | Bit Field | Description | Range |
| --- | --- | --- | --- |
| 7 | 0000 0011 | Opcode = Program table |  |
| 8 | 000p pppp | Internal Drumset selected for external program change 0 | (0-20) |
| 9 | 000p pppp | Internal Drumset selected for external program change 1 | (0-20) |
| - | ---- ---- | --- | --- |
| - | ---- ---- | --- | --- |
| 135 | 000p pppp | Internal Drumset selected for external program change 127 | (0-20) |
| 136 | 0xxx xxxx | Checksum | (0-127) |
| 137 | 1111 0111 | EOX |  |

Message 4 / Display Message

This message will display 32 characters on the LCDXE "LCD"§ for approximately 5 seconds and then restore the previous display.

Data Format (following header):

| Byte | Bit Field | Description | Range |
| --- | --- | --- | --- |
| 7 | 0000 0100 | Opcode = Display Message |  |
| 8 | 0aaa aaaa | LCDXE "LCD"§ character position 0 | (ascii) |
| 9 | 0aaa aaaa | LCDXE "LCD"§ character position 1 | (ascii) |
| --- | ---- ---- | --- | --- |
| --- | ---- ---- | --- | --- |
| 39 | 0aaa aaaa | LCDXE "LCD"§ character position 31 | (ascii) |
| 40 | 1111 0111 | EOX |  |

Message 5 / New Trigger Setup  (versions 1.02 and greater)

Note that the D5XE "D5"§ can both transmit and receive trigger setups to and from a D4XE "D4"§.

Data Format (following header):

| Byte | Bit Field | Description | Range |
| --- | --- | --- | --- |
| 7 | 0000 0101 | Opcode = New Trigger Setup |  |
| 8 | 0ggg gggg | Trigger 1 gain | (0-99) |
| 9 | 0ggg gggg | Trigger 2 gain | (0-99) |
| - | ---- ---- | --- | --- |
| - | ---- ---- | --- | --- |
| 18 | 0ggg gggg | Trigger 11 gain | (0-99) |
| 19 | 0ggg gggg | Trigger 12 gain | (0-99) |
| 20 | 0000 0ccc | Trigger 1 vcurve | (0-7) |
| 21 | 0nnn nnnn | Trigger 1 cross-talk | (0-99) |
| 22 | 0xxx xxxx | Trigger 1 noise floor | (0-99) |
| 23 | 0ddd dddd | Trigger 1 decay | (0-99) |
| 24 | 0000 0ccc | Trigger 2 vcurve | (0-7) |
| 25 | 0nnn nnnn | Trigger 2 cross-talk | (0-99) |
| 26 | 0xxx xxxx | Trigger 2 noise floor | (0-99) |
| 27 | 0ddd dddd | Trigger 2 decay | (0-99) |
| - | ---- ---- | --- | --- |
| - | ---- ---- | --- | --- |
| 64 | 0000 0ccc | Trigger 12 vcurve | (0-7) |
| 65 | 0nnn nnnn | Trigger 12 cross-talk | (0-99) |
| 66 | 0xxx xxxx | Trigger 12 noise floor | (0-99) |
| 67 | 0ddd dddd | Trigger 12 decay | (0-99) |
| 68 | 0xxx xxxx | Checksum | (0-127) |
| 69 | 1111 0111 | EOX |  |

Messages 32 thru 52 / Single Drumset Only

These messages contain the same Drumset data structures as the Edit Buffer message and are identical, with the exception of the opcode byte.  When a D4XE "D4"§ receives a complete Single Drumset message it will automatically "Store" the data in one of the 21 memory locations (as specified in the opcode) replacing the existing Drumset data for that slot.  Its important to note that this new Drumset data does not effect the edit buffer.  For a detailed listing of the Drumset data structure see the description under Message 1 / Edit Buffer.